# Lab 8: Static Sequential Memories and Tri-State Buses

Author: Seth Ricks

Date: 11/17/2025

Class: ECEN 340

Purpose:

- To learn how to design Static Memories

- To learn how to deal with tri-state (3-state) data buses

- To learn to trust simulation for sub modules

- To gain experience understanding Verilog code written by someone else.

Procedure:

Part 1.  Simulating a single 16 X 16 Static memory.

a) Design the 16 X 16 Static RAM module called "memory".  Note that the cell ports are: we, oe, data, addr,  and clk.  The data is accessed via a bi-directional 16-bit bus.  See class slides for details on memory arrays (class slides are duplicated below).

b) Write a test bench to verify functionality of the memory (class slides are duplicated below).

module memory(

input wire oe, we, clk,

input wire [14:0] addr,

inout wire [7:0] data

);

reg [7:0] mem [32767:0];

assign data = (oe && !we) ? mem[addr]:8’hzz;

always@(posedge clk)

begin

if (we) mem[addr] <= data;

end

endmodule

Note: The above code will synthesize a memory that is 32768 X 8bit.  Understand the difference between this memory and the 16 X 16bit memory of this lab.

module memory_tb( );

reg we, oe, clk;

wire [7:0] data;

reg [3:0] addr;

reg [7:0] data_temp;

parameter period = 10;

memory u0 (.we(we), .oe(oe), .clk(clk), .data(data), .addr(addr));

assign data = we?data_temp:8'hZZ;

always #(period/2) clk = ~clk;

initial

begin    clk = 0;    oe = 0;    we = 0; addr = 4’h0; data_temp = 8’h00;

#5 we = 1'b1;

@(posedge clk)

#1              addr = 4'h0; data_temp = 8'h34;   // wait 1 ns before writing

#period     addr = 4'h1; data_temp = 8'h58;        // writing to memory

#period     addr = 4'h2; data_temp = 8'hA5;

#period     addr = 4'h3; data_temp = 8'h5A;

…

#period     addr = 4'h0; we= 1'b0; oe = 1'b1;      // reading from memory

#period     addr = 4'h1;

#period     addr = 4'h2;

…

#1000 $finish

end

endmodule

Note: The above code is a test bench excerpt for the memory module.

Part 2. Implement the complete lab in Vivado. Homework 8 will serve as the foundation for understanding this lab. You will need:

1) Your four digit 7-segment display modules.

2) Your "memory" module

3) The pre-written modules provided.

You will not need to write a test bench for this part of the lab.  When the lab is completely implemented, you will observe that the top-level design instantiates two 16 X 16 SRAM cells, 5 push-button debouncers, and the seven-segment design of Lab 6.

Block Diagram of 3-state Memory Lab

Results/Report:

This entire lab was based on the premise that we need to be able to read and understand someone else’s code. It was difficult for me, and helped me appreciate how important comments are.

Part 1:

The quick fox jumps over the lazy dog. The quick fox jumps over the lazy dog. The quick fox jumps over the lazy dog.

Part 2:

The quick fox jumps over the lazy dog.

Conclusion:

The quick fox jumps over the lazy dog.