Homework 7 – Verilog State Machines (65 points)

1.	Use D-type flip-flops and combinational logic to design a synchronous Moore finite-state machine that monitors input “A” and asserts a binary output “B” if the sequence “101” is observed.  For example:

If the input data is:	A=010101101

The output will be:	B=000101001

------------------------------------- time

a) Draw the state graph (5 points)

b) Create the next state/output table using state names (5 points)

c) Make state assignments and substitute numbers for names in the next state table (5 points)

d) Determine the minimal circuit realization of the next state logic and output (5 points)

e) Draw the circuit (5 points)

f) Draw a timing diagram using the input sequence, A = 010101101.  Show the input, output, next state, and current state (5 points).

2.	Draw the modified State diagram of problem 1 for a Mealy-type State machine (5 points).

3.	Write the Verilog code for the above state machine using Moore-type outputs.  When writing the code, please do the following:

- Use parameters for the state names (5 points).  
For example: 
     parameter got1 = 2’b00;

- Use a case statement to determine the next state based on the current state (5 points).

- Place the state flip-flops in their own “always” block (5 points).

- Use a separate case statement for the “found” signal (5 points).

4. Modify the Moore-type output code for the Mealy-type outputs (10 points).